Annexure -IV

TECHNICAL SPECIFICATION FOR INSTRUMENTATION LAB

| SL No | Description | Quantity |
| --- | --- | --- |
| 1. | Modular Automation Production System 6 STATION 1: DISTRIBUTION STATION Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform Dispensing Module: The work piece is dispensed from the stacking Rotary Pick and Place(RPP) Module: The rotary arm picks up the work piece from the station and places it in the next station for testing operations I/O Terminal is provided on the operating console with plug in wires Operating console with cycle Start/Stop, emergency stop buttons are provided for easy control. Power Supply Unit with Input Voltage: 115/230V Output Voltage: 24V DC Output Current: 4.5A with short circuit protection Fully wired & Programmed PLC Unit is provided with a) Siemens PLC, b) RS232 Comm. Cable, c) PLC Programming Software Service Unit: FRL Unit Technical Literature: Exercise Manual - Training Document for Distribution Station Reference manual Accessories: 1. 1 Unit Ethernet Cable 2. 1 Unit Work piece (21 pieces) for assembly. Work piece is cylindrical bottle STATION 2: TESTING STATION Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform. Sensing Module - To identify the nature of Work piece. An inductive sensor is used to identify the metal and a Capacitive sensor is used to identify non metal. Height Measuring Module - To measure the height of the work piece using an LVDT. The measured value is processed by a PLC. Maximum height between measurement: 50mm. Rejection Module: It is used to reject the work piece. Conveyor Module: Transfers the work piece from LVDT module to RPP module. RPP Module: The rotary arm picks up the work piece from the station and places it in the next station I/O Terminal is provided on the operating console with plug-in wires. Operating console with cycle Start/Stop, emergency stop buttons are provided for easy control. Power Supply Unit with Input Voltage: 115/230V Output Voltage: 24V DC Output Current: 4.5A with short circuit protection Fully wired & Programmed PLC Unit is provided with a) Siemens PLC b) RS232 Comm. Cable c) PLC Programming Software Service Unit: FRL Unit Technical Literature: Exercise Manual - Training Document for Testing Station. Reference Manual Accessories: 1. 1 Unit Ethernet Cable STATION 3. PROCESSING STATION Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform Rotary Indexing Module: 6 Station Rotary Indexing Table for holding Workpiece. Capacitive sensor for detecting the presence of Workpiece is provided. Filling Module 1: The Filling of the bottle with coloured beads is executed here. Filling Module 2: The Filling of the bottle with coloured beads is executed here. Inspection Module - This verifies the presence of a bead in the cylinder. Option- Vision Inspection System can be ordered. Rotary Pick & Place Module 1: It is used to reject the unfilled bottles after inspection. Rotary Pick & Place Module 2: It is used to transfer the workpiece to the next station. I/O Terminal is provided on the operating console with plug-in wires Operating console with cycle Start/Stop, emergency stop buttons are provided for easy control. Power Supply Unit with Input Voltage: 115/230V Output Voltage: 24V DC Output Current: 4.5A with short circuit protection Fully wired & Programmed PLC Unit is provided with a) Siemens PLC b) RS232 Comm. Cable c) PLC Programming Software Service Unit: FRL Unit Technical Literature: Exercise Manual - Training Document for Processing Station Reference Manual Accessories: - 1 Unit Ethernet Cable COMMON FOR STATION 4 & 5: Transfer Module and Shuttle Arm: This unit consists of a base and a shuttle arm which carries forward the workpiece to 4 positions. The first position is the workpiece receiving station; the shuttle arm next carries it to the Colour detection station, followed by Capping station and RPP station. STATION 4: COLOUR DETECTION STATION Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform. Colour Detecting Module: Colour sensor detects the colour of the filled material. Depending on the colour of the filled material signal is sent to the XY Palletizer to arrange the test piece accordingly. Shuttle Conveyor Module: Stepper motor is used move the shuttle to different positions as described above. STATION 5: ASSEMBLY STATION Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform Capping Module: The bottle is capped after the colour detection module. Rotary Pick & Place Module: The 2 axes rotary pick and place module is used to transfer the workpiece to the next station Operating console with cycle Start/Stop, emergency stop buttons are provided for easy control. Power Supply Unit with Input Voltage: 115/230V Output Voltage: 24V DC Output Current: 4.5A with short circuit protection Service Unit - FRL Unit Fully wired & Programmed PLC Unit is provided with a) Siemens PLC b) RS232 Comm. Cable c) PLC Programming Software Technical literature Reference manual, Exercise Manual- Training document for Assembly Station Accessories: 1. 1 Unit Ethernet Cable STATION 6: STORAGE MODULE Aluminium Profile Plate of dimension 400x700mm for mounting all pneumatic & other components to the modules. A Mobile trolley which can fit the aluminium profile plate (400x700mm) is provided. All electrical and electronic items shall be housed in the bottom of the trolley below the platform Weighing Module: This module employees a load cell to weigh the bottle and depending on the weight of the bottle signal is sent to the XY Palletizer to arrange the test piece accordingly. XY-Palletizer Module: After weighing the assembled cylindrical bottle is palletized where the Palletizer gripper picks up the workpiece for storage. For positioning the workpiece in an array with 2 axis controller. Stepper motors drive the axes. Position control is achieved using these motors. The axes work on a gantry-axis manipulator concept. I/O Terminal is provided on the operating console with plug-in wires Operating console with cycle Start/Stop, emergency stop buttons are provided for easy control. Power Supply Unit with Input Voltage: 115/230V Output Voltage: 24V DC Output Current: 4.5A with short circuit protection Fully wired & Programmed PLC Unit is provided with a) Siemens PLC b) RS232 Comm. Cable c) PLC Programming Software Service Unit: FRL Unit Technical Literature: Exercise Manual - Training Document for Palletizing Station, Reference Manual Accessories: 1. 1 Unit Ethernet cable STATION 7: 6 AXIS ARTICULATED BRAND NAMED ARISTRO 6XT ROBOT Performance Features: Payload : 2.5 Kgs Tip Speed : 0.2 Meters /sec Repeatability : + / - 0.3mm MECHANICAL: Configuration : Vertical Articulated Five-bar linkage No. of Axes : 6(3 axes waist-shoulder-elbow manipulator with 3 axes Roll-Pitch- Roll spherical wrist) Link 1 300 mm Link 2 400 mm Vertical Height 400mm (450mm with controller box) Joint Actuators DC Servo geared motors Transmission Joint 1 : Worm Wheel Drive Joint 2 & 3 : Ball screw Joint 4,5 & 6 : Belt Drive Gravity Compensation : 100% (Non-back drivable ball screw) Position Feedback : Optical Encoder (HP 2 phase 500 ppr) Gripper : Pneumatic Joint Motion Manipulator Waist : 300 degrees, Shoulder: 60 degrees, Elbow : 60 degrees Wrist Roll : 330 degrees, Pitch: 330 degrees, Roll : 330 degrees CONTROL / SOFTWARE PC based Controller PID Control algorithm Support for 2 additional axes excluding gripper. SENSORS (MANIPULATOR): Datum Home sensors for each axis and software limits Proximity home sensors. CONTROL SOFTWARE: Specially developed Robot Programming Language ACCESSORIES & INPUT / OUTPUT: 2 servo axis each with 3 digital inputs and one output. 16 digital Inputs/Outputs (Optional) POWER SUPPLY: 230 V AC, 50/60 Hz , (+5V @ 25A, 12V @ 8A) 2 YEARS Demonstration and sufficient training (free) to the faculty and the student will be given | 01 No. |
| 2. | Flow meter Trainer for Flow measurement using different Sensors & following accessories - Rotameter with Transducer - Electromagnetic Flow Meter - Ultrasonic Flow Meter - Venturi Nozzle - Pitot Tube - Orifice Plate Flow Meter with Transducer Flow meter Trainer Learning Objectives / Experiments together with different flow meters available as accessories - different flow meters and their principles of operation - calibration of different flow meters - position dependency of flow meters - plotting and comparison of pressure loss curves Specification [1] comparison and calibration of different flow meters [2] water circuit with tank, pump and valve to adjust flow rate [3] 2 measuring locations for vertical or horizontal installation of the flow meters under test [4] electromagnetic flow rate sensor for reference measurement [5] 1 differential pressure sensor and twin tube manometers for measurement of pressure losses [6] DC voltage source to supply the flow meters with auxiliary power [7] flow meters available as accessories Technical Data Tank: approx. 55L Pump - max. flow rate: approx. 225L/min - max. head: approx. 11m DC voltage source - voltage: 24VDC - current: 2,0A Accuracy of electromagnetic flow rate sensor - 0.5% of final value Measuring ranges - flow rate (reference): 0...4760L/h - differential pressure sensor: 0...2bar - twin tube manometers: 0...680mmWC Along with the following accessories: - Rotameter with Transducer - Electromagnetic Flow Meter Learning Objectives / Experiments - familiarisation with the principle of operation - flow measurement - plotting a pressure loss curve - comparison with other flow meters Specifications: [1] electromagnetic flow meter as accessory to flow meter trainer [2] operates according to Faraday's Law of Induction [3] display indicating flow rate [4] connections to facilitate pressure loss measurement with the flow meter trainer [5] connections to supply auxiliary power via the flow meter trainer [6] vertical and horizontal installation possible Technical data Max. flow rate: 4760L/h Auxiliary power: 24VDC Pipe connections: DN 32 - Ultrasonic Flow Meter Learning Objectives / Experiments - familiarisation with the principle of operation - flow measurement - plotting a pressure loss curve - comparison with other flow meters Specification [1] ultrasonic flow meter as accessory for flow meter trainer [2] operates according to principle of transit-time measurement [3] display indicating flow rate [4] connections to facilitate pressure loss measurement with the flow meter trainer [5] battery to supply auxiliary power [6] vertical and horizontal installation possible Technical Data Measuring range: 35...7000L/h Pipe connections: DN 32 - Venturi Nozzle Learning Objectives / Experiments - familiarisation with the principle of operation * Bernoulli's principle * continuity law - flow rate measurement - plotting a pressure loss curve - comparison with other flow meters Specifications: [1] venturi nozzle for flow rate measurement as accessory for trainer flow meter trainer [2] operation based on the differential pressure method [3] venturi nozzle with transparent front [4] display of pressure drop for calculation of flow rate via flow meter trainer [5] connections to facilitate pressure loss measurement with the flow meter trainer [6] vertical and horizontal installation possible Technical data: Venturi nozzle - cross-section upstream of constriction 18,4x18,4mm - cross-section at constriction: 4,6x18,4mm - material: PVC and PMMA Pipe connections: DN 32 - Pitot Tube Learning Objectives / Experiments - familiarisation with the principle of operation * dynamic and static pressure * continuity law and Bernoulli's principle - flow rate measurement - plotting a pressure loss curve - comparison with other flow meters Specification [1] Pitot tube for flow rate measurement as accessory for trainer flow meter trainer [2] Calculation of flow rate obtained from difference between total pressure (pitot tube) and static pressure (vertical pipe) [3] Display of pressure difference via flow meter trainer [4] Connections to facilitate pressure loss measurement with the flow meter trainer [5] Meter housing made from transparent material [6] Vertical and horizontal installation possible Technical Data Pitot tube and vertical pipe - material: brass - inside diameter: approx. 2mm Pipe connections: DN 32 - Orifice Plate Flow Meter with Transducer Learning Objectives / Experiments - familiarisation with the operating principle * continuity law and Bernoulli's principle * transducer with capacitive sensing element - flow rate measurement - plotting a pressure loss curve - comparison with other flow meters Specifications: [1] orifice Plate Flow Meter with electronic differential pressure transducer for flow rate measurement as accessory for trainer flow meter trainer [2] operation based on the differential pressure method [3] display indicating differential pressure [4] connections to facilitate pressure loss measurement with the flow meter trainer [5] connections to supply auxiliary power via the flow meter trainer [6] vertical and horizontal installation possible Technical data Orifice plate - material: brass - diameter: 18,5mm, with 45° chamfer Transducer measuring range: 0...500mbar Auxiliary power: 24VDC Pipe connections: DN 32 | 01 No |